Universidad Hispanoamericana

Proyecto Programado Teleproceso y Redes

Estudiante:

Alex Joan Roblero Quiros

Profesor:

Milton Emilio Vargas Jiménez

Teleproceso y Redes

Abril 2026

Contenido

Topología:	3

Configuraciones:	3

Router San José:	3

Show running:	3

Router Cartago:	10

Show running:	10

Switch San José:	16

Show running:	16

Show vlan brief:	24

Switch Cartago:	25

Show running:	25

Show vlan brief:	32

Tabla de IP:	33

Pruebas de Conectividad:	34

Prueba de conexión desde la PC - Administración (San José) hacia la PC - Profesores (Cartago)	34

Prueba de conexión desde la PC - Academia (San José) hacia la PC - Laboratorio (San José)	35

Prueba de búsqueda de la página web desde la PC - Estudiantes (Cartago)	36

Verificación de datos automáticos en la PC - Profesores (Cartago)	37

# Topología:

# Configuraciones:

## Router San José:

### Show running:

R-SanJose#show running

Building configuration...

Current configuration : 1458 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R-SanJose

!

!

!

!

ip dhcp excluded-address 10.100.20.1

ip dhcp excluded-address 10.100.20.126

ip dhcp excluded-address 10.100.20.129

ip dhcp excluded-address 10.100.21.1

!

ip dhcp pool vlan10

network 10.100.20.0 255.255.255.128

default-router 10.100.20.1

dns-server 10.100.20.126

ip dhcp pool vlan20

network 10.100.20.128 255.255.255.192

default-router 10.100.20.129

dns-server 10.100.20.126

ip dhcp pool vlan30

network 10.100.21.0 255.255.255.0

default-router 10.100.21.1

dns-server 10.100.20.126

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO1941/K9 sn FTX1524JH93-

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 10.100.20.1 255.255.255.128

!

interface GigabitEthernet0/0.20

encapsulation dot1Q 20

ip address 10.100.20.129 255.255.255.192

!

interface GigabitEthernet0/0.30

encapsulation dot1Q 30

ip address 10.100.21.1 255.255.255.0

!

interface GigabitEthernet0/1

ip address 10.100.44.1 255.255.252.0

duplex auto

speed auto

!

interface Vlan1

no ip address

shutdown

!

router rip

version 2

network 10.0.0.0

no auto-summary

!

ip classless

!

ip flow-export version 9

!

!

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

end

## Router Cartago:

### Show running:

R-Cartago#show running

Building configuration...

Current configuration : 1446 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R-Cartago

!

!

!

!

ip dhcp excluded-address 10.100.22.1

ip dhcp excluded-address 10.100.23.254

ip dhcp excluded-address 10.100.24.1

ip dhcp excluded-address 10.100.28.1

!

ip dhcp pool vlan40

network 10.100.22.0 255.255.254.0

default-router 10.100.22.1

dns-server 10.100.20.126

ip dhcp pool vlan50

network 10.100.24.0 255.255.255.192

default-router 10.100.24.1

dns-server 10.100.20.126

ip dhcp pool vlan60

network 10.100.28.0 255.255.252.0

default-router 10.100.28.1

dns-server 10.100.20.126

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO1941/K9 sn FTX1524DTS6-

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/0.40

encapsulation dot1Q 40

ip address 10.100.22.1 255.255.254.0

!

interface GigabitEthernet0/0.50

encapsulation dot1Q 50

ip address 10.100.24.1 255.255.255.192

!

interface GigabitEthernet0/0.60

encapsulation dot1Q 60

ip address 10.100.28.1 255.255.252.0

!

interface GigabitEthernet0/1

ip address 10.100.44.2 255.255.252.0

duplex auto

speed auto

!

interface Vlan1

no ip address

shutdown

!

router rip

version 2

network 10.0.0.0

no auto-summary

!

ip classless

!

ip flow-export version 9

!

!

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

end

## Switch San José:

### Show running:

Sw-SanJose#show running

Building configuration...

Current configuration : 2127 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname Sw-SanJose

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/2

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/3

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/4

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/5

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/6

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/7

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/8

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/9

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/10

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/11

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/12

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/13

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/14

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/15

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/16

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/17

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/18

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/19

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/20

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport mode trunk

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

!

!

!

line con 0

!

line vty 0 4

login

line vty 5 15

login

!

!

!

!

End

### Show vlan brief:

Sw-SanJose#show vlan brief

VLAN Name Status Ports

---- -------------------------------- --------- -------------------------------

1 default active Fa0/21, Fa0/22, Fa0/23, Fa0/24

Gig0/2

10 Administracion active Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6, Fa0/7, Fa0/8

20 Academicas active Fa0/9, Fa0/10, Fa0/11, Fa0/12

Fa0/13, Fa0/14, Fa0/15

30 Laboratorio active Fa0/16, Fa0/17, Fa0/18, Fa0/19

Fa0/20

1002 fddi-default active

1003 token-ring-default active

1004 fddinet-default active

1005 trnet-default active

## Switch Cartago:

### Show running:

Sw-Cartago#show running

Building configuration...

Current configuration : 2127 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname Sw-Cartago

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/2

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/3

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/4

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/5

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/6

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/7

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/8

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/9

switchport access vlan 50

switchport mode access

!

interface FastEthernet0/10

switchport access vlan 50

switchport mode access

!

interface FastEthernet0/11

switchport access vlan 50

switchport mode access

!

interface FastEthernet0/12

switchport access vlan 50

switchport mode access

!

interface FastEthernet0/13

switchport access vlan 50

switchport mode access

!

interface FastEthernet0/14

switchport access vlan 50

switchport mode access

!

interface FastEthernet0/15

switchport access vlan 50

switchport mode access

!

interface FastEthernet0/16

switchport access vlan 60

switchport mode access

!

interface FastEthernet0/17

switchport access vlan 60

switchport mode access

!

interface FastEthernet0/18

switchport access vlan 60

switchport mode access

!

interface FastEthernet0/19

switchport access vlan 60

switchport mode access

!

interface FastEthernet0/20

switchport access vlan 60

switchport mode access

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport mode trunk

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

!

!

!

line con 0

!

line vty 0 4

login

line vty 5 15

login

!

!

!

!

end

### Show vlan brief:

Sw-Cartago#show vlan brief

VLAN Name Status Ports

---- -------------------------------- --------- -------------------------------

1 default active Fa0/21, Fa0/22, Fa0/23, Fa0/24

Gig0/2

40 Profesores active Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/6, Fa0/7, Fa0/8

50 Estudiantes active Fa0/9, Fa0/10, Fa0/11, Fa0/12

Fa0/13, Fa0/14, Fa0/15

60 Servicio active Fa0/16, Fa0/17, Fa0/18, Fa0/19

Fa0/20

1002 fddi-default active

1003 token-ring-default active

1004 fddinet-default active

1005 trnet-default active

# Tabla de IP:

| VLAN | Departamento | Hosts | Dirección de Red | Máscara de Subred | Gateway (Router IP) |
| --- | --- | --- | --- | --- | --- |
| 10 | Administración | 100 | 10.100.20.0 | 255.255.255.128 | 10.100.20.1 |
| 20 | Académicas | 50 | 10.100.20.128 | 255.255.255.192 | 10.100.20.129 |
| 30 | Laboratorio | 250 | 10.100.21.0 | 255.255.255.0 | 10.100.21.1 |
| 40 | Profesores | 500 | 10.100.22.0 | 255.255.254.0 | 10.100.22.1 |
| 50 | Estudiantes | 60 | 10.100.24.0 | 255.255.255.192 | 10.100.24.1 |
| 60 | Servicio | 1000 | 10.100.28.0 | 255.255.252.0 | 10.100.28.1 |

# Pruebas de Conectividad:

Prueba de conexión desde la PC - Administración (San José) hacia la PC - Profesores (Cartago)

Aquí estoy probando que los datos viajan sin problemas entre las dos sedes. Como se ve en el ping, la comunicación entre San José y Cartago funciona correctamente gracias a la ruta que configuré en los routers.

## Prueba de conexión desde la PC - Academia (San José) hacia la PC - Laboratorio (San José)

Con este ping confirmo que los diferentes departamentos de la sede San José pueden hablarse entre ellos. Esto demuestra que el enrutamiento interno que hice en el router está trabajando bien.

## Prueba de búsqueda de la página web desde la PC - Estudiantes (Cartago)

En esta parte verifiqué que la computadora de los estudiantes puede encontrar el servidor de la universidad usando solo el nombre www.uh.com. Esto confirma que el servidor DNS está traduciendo la dirección correctamente.

## Verificación de datos automáticos en la PC - Profesores (Cartago)

Entré a la configuración de esta PC para asegurar que el sistema le asignó su dirección IP y el resto de los datos de forma automática. Como se ve en la captura, el DHCP le dio toda la información necesaria para navegar.